# PLC Connection Guide

TN011D-E

VNC function

# VNC (Virtual Network Computing) Introduction

VNC is a technology for remote desktop sharing. VNC enables the desktop/laptop display of one device to be remotely viewed and controlled over a network connection. This technology is useful in control center; it will allow someone to access their device from another place of their desktop or laptop. It is also useful for network administrators in business environments.

VNC works similarly to the Remote Desktop appplication built into newer versions of Microsoft Windows. Unlike Windows Remote Desktop, VNC runs on older Windows computers, Linux/Unix and other non-Windows operating systems. VNC applications, however, are generally regarded as slower and offering fewer features and security options than Windows Remote Desktop.

# VNC Setting

| Software: | WebOP Designer v2.0 (02-09th-2012) |
| --- | --- |
| Hardware: | T Series with Ethernet port model |
| Operation | HMI Setting - When you enter the Panel Setup mode, you will see the FTP/VNC button.<Fig.A-1> <Fig. A-1> - Under FTP/VNC option, please follow up below setting to enable the VNC function: Enable: True Lower User Level For Control: 8 (User Level needs Level to Control) Lower User Level For Monitor: 8 (User Level needs Level to Monitor) - Under general option, please follow up below setting to define the IP Address & Subnet Mask: <Fig. A-2> VNC Tool Setting Please try to download the free “realVNC” software and install it into your desktop and laptop. When you install finish, please executing the VNC Viewer 4 and then you will see below dialog box。 Setting of the VNC Viewer 4 Server：Please key-in the IP Address of the HMI device。 Note: If you forgot the IP address of the HMI, please go to recheck with the <Fig. A-2> to know how can you get the IP Address from the HMI hardware. If the IP Address is correct, you will enter the Authentication dialog box. Please see below setting: Username：Blank Password：Please recheck with the password from the HMI software, and then key-in the password of the User Level 8。 Please see below picture. <Fig. A-3> <Fig. A-3> |